MonSter: Marry Monodepth to Stereo Unleashes Power

Junda Cheng1 ,   Longliang Liu1 ,   Gangwei Xu1 ,   Xianqi Wang1 ,

Zhaoxing Zhang1 ,   Yong Deng2 ,   Jinliang Zang2 ,   Yurui Chen2 ,   Zhipeng Cai3 ,   Xin Yang1†

1 Huazhong University of Science and Technology     2  Autel Robotics     3  Intel Labs

{jundacheng,  longliangl,  gwxu,  xianqiw,  zzx,  xinyang2014}@hust .edu .cn,  czptc2h@gmail .com

arXiv:2501.08643v1  [cs.CV]  15 Jan 2025

MonSter(Ours)       IGEV             Left Image

Figure 1.  Zero-shot generalization comparison:  all models are trained on Scene Flow and tested directly on KITTI. Compared to the baseline IGEV [38], our method MonSter shows significant improvement in challenging regions such as reflective surfaces, textureless areas, fine structures, and distant objects.

Abstract

Stereo matching recovers depth from image correspon- dences.     Existing  methods  struggle  to  handle  ill-posed regions  with  limited  matching  cues,  such  as  occlusions and textureless areas.   To address  this, we propose Mon- Ster,  a  novel  method  that  leverages  the  complementary strengths of monocular depth estimation and stereo match- ing. MonSter integrates monocular depth and stereo match- ing into a dual-branch architecture to iteratively improve each other.  Confidence-based guidance adaptively selects reliable stereo cuesfor monodepth scale-shift recovery. The refined monodepth is in turn guides stereo effectively at ill- posed regions.  Such iterative mutual enhancement enables MonSter to evolve  monodepth priors from coarse object- level structures to pixel-level geometry, fully unlocking the potential of stereo matching.   As shown in Fig. 2, Mon- Ster ranks 1st  across five most commonly used leaderboards —  SceneFlow, KITTI 2012, KITTI 2015, Middlebury, and ETH3D. Achieving  up  to 49.5%  improvements  (Bad  1.0

† Corresponding author.

on ETH3D) over the previous best method.   Comprehen- sive analysis verifies  the  effectiveness of MonSter in  ill- posed regions.  In terms of zero-shot generalization, Mon- Ster significantly and consistently outperforms state-of-the- art across the board.    The code is publicly available at: https://github.com/Junda24/MonSter.

# 1. Introduction

Stereo matching estimates a disparity map from rectified stereo images, which can be subsequently converted into metric depth.  It is the core of many applications such as self-driving, robotic navigation, and 3D reconstruction.

Deep learning based methods [4, 14, 16, 20, 35, 48] have demonstrated promising performance on standard bench- marks. These methods can be roughly categorized into cost filtering-based  methods  and  iterative  optimization-based methods.  Cost filtering-based methods  [4, 14, 15, 22, 37, 39, 41, 43] construct 3D/4D cost volume using CNN fea- tures, followed by a series of 2D/3D convolutions for regu- larization and filtering to minimize mismatches.  Iterative

Global Scale + Global Shift × Per-Pixel Shift          ×

Global Scale + Global Shift Per-Pixel Shift

Sceneflow (EPE)

KITTI 15

(D1-bg)

(Out-3)

KITTI 12

Los IGEV CREStereo NMRF-Stereo Selective-IGEV MonSter (Ours)

ETH3D (Bad 1.0)                   Middlebury (RMSE)

Figure 2.   Leaderboard  performance.   Our  method ranks  1st across 5 leaderboards, advancing SOTA by a large margin.

optimization-based methods  [10, 16, 20, 38, 49]  initially construct an all-pairs correlation volume, then index a lo- cal cost to extract motion features, which guide the recur- rent units (ConvGRUs) [9] to iteratively refine the disparity map.

Both types of methods essentially derive disparity from similarity matching, assuming visible correspondences in both  images.      This  poses  challenges  in  ill-posed  re- gions  with  limited  matching  cues,  e.g.,  occlusions,  tex- tureless  areas,  repetitive/thin  structures,  and  distant  ob- jects with low pixel representation.  Existing methods [4, 31, 33, 37, 42, 44, 49]  address  this  issue  by  enhanc- ing  matching  with  stronger  feature  representations  dur- ing feature extraction or cost aggregation:  GMStereo [42] and STTR [18] employ transformer as feature extractors; IGEV [38] and ACVNet [37] incorporate geometric infor- mation into the cost volume using attention mechanisms to strengthen matching information;  Selective-IGEV  [35] and DLNR [49] further improve performance by introduc- ing high-frequency information during the iterative refine- ment stage.  However, these methods still struggle to fun- damentally resolve the issue of mismatching, limiting their practical performance.

Unlike stereo matching, monocular depth estimation di- rectly recovers 3D from a single image, which does not en- counter the challenge of mismatching.   While monocular depth provides complementary  structural information for stereo, pre-trained models often yield relative depth with scale and shift ambiguities. As shown in Fig.3, the predic- tion of monodepth models differ heavily from the ground- truth.   Even  after  global  scale  and  shift  alignment,  sub- stantial errors still persist, complicating pixel-wise fusion of monocular depth and stereo disparity.   Based on these insights, we propose MonSter, a novel approach that de- composes stereo matching into monocular depth estimation and per-pixel  scale-shift recovery,  which  fully  combines the strengths of monocular and stereo algorithms and over- comes the limitations from the lack of matching cues.

MonSter  constructs  separate  branches  for  monocular

400

300

200

100

0

20            40            60

| Global Scale + Global Shift Per-Pixel Shift |  |
| --- | --- |

Disparity derived from Monodepth

Disparity derived from Monodepth

Disparity derived from Monodepth

60

50

40

30

20

10

10    20    30    40    50    60

10            30            50  口

GT Disparity

GT Disparity

(b) Align with GT

60

50

40

30

20

10

10    20    30    40    50    60

GT Disparity

(c) Our Shift Refinement

(a) No Alignment

Figure 3. Distance between GT disparity and the disparity de- rived from monocular depth [46] on KITTI dataset.  The red line indicates identical disparity maps.   (a):  Without  any align- ment.   (b)  Align  depth  with GT using global  scale and global shift values (same for all pixels). (c) The monocular disparity pro- duced by MonSter, with per-pixel shift refinement. Even globally aligned with GT, SOTA monocular depth models still exhibit se- vere noise.  Our method MonSter effectively addresses this issue by refining monocular depth with per-pixel shift, which fully un- locks the power of monocular depth priors for stereo matching.

depth estimation and stereo matching, and adaptively fuses them through Stereo Guided Alignment (SGA) and Mono Guided Refinement  (MGR) modules.   SGA  first  rescales monodepth into a “monocular disparity” by aligning glob- ally with stereo disparity.   Then it uses condition-guided GRUs to adaptively select reliable stereo cues for updat- ing the per-pixel monocular disparity shift.  Symmetric to SGA, MGR uses the optimized monocular disparity as the condition to adaptively refine the stereo disparity in regions where matching fails.  Through multiple iterations, the two branches effectively complement each other:   1)  Though beneficial at coarse object-level, directly and unidirection- ally fuse monodepth into stereo suffers from scale-shift am- biguities, which often introduces noise in complex regions such as slanted or curved surfaces.   Refining monodepth with stereo resolves this issue effectively, ensuring the ro- bustness of MonSter.   2)  The refined monodepth in turn provides strong guidance to stereo in challenging regions. E.g.,  the depth perception ability of stereo matching de- grades with distance due to smaller pixel proportions and the increased matching difficulty.  Monodepth models pre- trained on large-scale datasets are less affected by such is- sues, which can effectively improve stereo disparity in the corresponding region.

Our main contributions can be summarized as follows:

•  We propose a novel  stereo-matching method MonSter, fully leveraging the pixel-level monocular depth priors to significantly improve the depth perception performance of stereo matching in ill-posed regions and fine structures.

•  As shown in Fig. 2, MonSter ranks 1st  across 5 widely- used leaderboards:  KITTI 2012 [11], KITTI 2015 [23], Scene Flow [22], Middlebury [27] and ETH3D [28]. Ad-

vancing SOTA by up to 49.5%.

•  Compared  to   SOTA  methods,   MonSter   achieves  the best zero-shot generalization consistently across diverse datasets.     MonSter  trained  solely  on  synthetic  data demonstrates  strong  performance  across  diverse  real- world datasets (see Fig.1).

# 2. Related Work

Matching based Methods.   Mainstream stereo matching methods  recover  disparity  from  matching  costs.    These methods can generally be divided into two categories: cost filtering-based  methods  and  iterative  optimization-based methods. Cost filtering-based methods [5, 6, 12, 14, 22, 29, 43] typically construct a 3D/4D cost volume using feature maps and subsequently employing 2D/3D CNN to filter the volume and derive the final disparity map.  Constructing a cost volume with strong representational capacity and ac- curately regressing disparity from a noisy cost volume are key challenges.  PSMNet [4] proposes a stacked hourglass 3D CNN for better cost regularization. [14, 30, 37] propose the group-wise correlation volume, the attention concatena- tion volume, and the pyramid warping volume respectively to improve the expressiveness of the matching cost.   Re- cently, a novel class of methods based on iterative optimiza- tion [16, 20, 35, 38, 49] has achieved state-of-the-art perfor- mance in both accuracy and efficiency.  These methods use ConvGRUs to iteratively update the disparity by leveraging local cost values retrieved from the all-pairs correlation vol- ume.  Similarly, iterative optimization-based methods also primarily focus on improving the matching cost construc- tion and the iterative optimization stages.  CREStereo [16] proposes a cascaded recurrent network to update the dispar- ity field in a coarse-to-fine manner.   IGEV [38] proposes a Geometry Encoding Volume that encodes geometry and context information for a more robust matching cost.  Both types of methods essentially derive depth from matching costs, which are inherently limited by ill-posed regions.

Stereo Matching with Structural Priors. Matching in ill- posed regions is challenging, previous methods [7, 17, 19, 31, 44, 47] tried to leverage structural priors to address this issue.  EdgeStereo  [31] enhances performance in edge re- gions by incorporating edge detection cues into the disparity estimation pipeline.  SegStereo [44] utilizes semantic cues from a segmentation network as guidance for stereo match- ing, improving performance in textureless regions.  How- ever, semantic and edge cues only provide object-level pri- ors, which are insufficient for pixel-level depth perception. Consequently, they are not effective in challenging scenes with  large  curved  or  slanted  surfaces.    Therefore,  some methods incorporate monocular depth priors to provide per- pixel guidance for stereo matching through dense relative depth.  CLStereo  [47] introduces a monocular branch that serves as a contextual constraint, transferring geometric pri-

ors from the monocular to the stereo branch. Los [17] uses monocular depth as local structural prior to generating the slant plane, which can explicitly leverage structure infor- mation for updating disparities. However, monocular depth estimation suffers from severe scale and shift ambiguities as shown in Fig.3.  Directly using it as a structural prior to constrain stereo matching can introduce heavy noise.  Our MonSter adaptively selects reliable stereo disparity to cor- rect the scale and shift of monodepth, which fully leverages the monocular depth priors while avoiding noise, thereby significantly enhancing stereo performance in ill-posed re- gions.

# 3. Method

As shown in Fig. 4, MonSter consists of 1) a monocular depth branch, 2) a stereo matching branch and 3) a mu- tual refinement module.  The two branches estimate initial monocular depth and the stereo disparity, which are fed into mutual refinement to iteratively improve each other.

## 3.1. Monocular and Stereo Branches

The monocular depth branch can leverage most monocu- lar depth models to  achieve non-trivial performance im- provements (see Sec.4.5 for analysis).  Our best empirical configuration uses pretrained DepthAnythingV2 [46] as the monocular depth branch, which uses DINOv2 [24] as the encoder and DPT [26] as the decoder.  The stereo match- ing branch follows IGEV [38] to obtain the initial stereo disp, with modifications only to the feature extraction com- ponent, as shown in Fig. 4. To efficiently and fully leverage the pretrained monocular model, the stereo branch shares the ViT encoder in DINOv2 with the monocular branch, with parameters frozen to prevent the stereo matching train- ing from affecting its generalization ability. Moreover, the ViT architecture extracts feature at a single resolution, while recent stereo matching methods commonly utilize multi- scale features at four scales (1/32,  1/16,  1/8,  and  1/4  of the original image resolution ).  To fully align with IGEV, we  employ  a  stack  of 2D  convolutional  layers,  denoted as the feature transfer network, to downsample and trans- form the ViT feature into a collection of pyramid features F = {F0, F1, F2, F3 }, where Fk  ∈ R  × 25W-k  ×ck .  We follow IGEV to construct Geometry Encoding Volume and use the same ConvGRUs for iterative optimization. To bal- ance accuracy and efficiency, we perform only N1 iterations to obtain a initial stereo disparity with reasonable quality.

## 3.2. Mutual Refinement

Once the initial (relative) monocular depth and stereo dis- parity  are  obtained,  they  are fed into the mutual refine- ment module to iteratively refine each other.  We first per- form global scale-shift alignment, which converts monoc- ular depth into a disparity map and aligns it coarsely with

Stereo Guided Alignment

x N2

Metric Mono Disp

L

Lookup Freeze

Condition

Guided GRU

L

Relative Depth

Refined Mono disp      Cost

Mono Encoder

Stereo

Condition

Mono Decoder

∆shift

Left Image

Global Scale- Shift Alignment

Warping &

L

Concat

x N1

L

Cost

Confidence Feature

Mono Encoder

Mono

∆disp

Condition

Final Disp

Refined Stereo disp

Cost

Volume

Feature Transfer

∆disp

Image Pair

Init Stereo Disp

GRU

Mono Guided Refinement

Figure 4.  Overview of MonSter.  MonSter consists of a monocular depth estimation branch, a stereo matching branch, and a mutual refinement module. It iteratively improves one branch with priors from the other, effectively resolving the ill-posedness in stereo matching.

stereo outputs. Then we iteratively perform a dual-branched refinement: Stereo guided alignment (SGA) leverages stereo cues to update the per-pixel shift of the monocular dispar- ity; Mono guided refinement (MGR) leverages the aligned monocular prior to further refine stereo disparity.

Global  Scale-Shift  Alignment.      Global  Scale-Shift Alignment  performs  least  squares  optimization  over  a global scale sG  and a global shift tG  to coarsely align the inverse monocular depth with the stereo disparity:

D0M = sG DM  + tG ,

where DM (i) and D0S(i) are the inverse monocular depth and stereo disparity at the i-th pixel.  Ω represents the re- gion where stereo disparity values fall between 20% to 90% sorting in ascending order, which helps to filter unreliable regions such as the sky, extremely distant areas, and close- range outliers.   Intuitively,  this  step  converts  the  inverse monocular depth into a disparity map coarsely aligned with the stereo disparity, enabling effective mutual refinement in the same space. We call this aligned disparity map D0M the monocular disparity in the remainder of the paper.

Stereo  Guided  Alignment  (SGA).  Though  coarsely aligned, a unified shift tG  is not sufficient to recover accu- rate monocular disparity at different pixels. To fully release the potential of monocular depth prior, SGA leverages in- termediate stereo cues to further estimate a per-pixel resid- ual shift. To avoid introducing noisy stereo cues, SGA uses confidence based guidance. In each update step j, we com-

pute the confidence using the flow residual map FjS, which

is obtained by warping and subtracting features based on

the stereo disparity DjS  as:

FjS x, y) - F      (2)

where FL3 ,FR3  represent the quarter-resolution features of

left and right images in F respectively.  For each iteration,

we also use the current stereo disparity DjS  to index from

the Geometry Encoding Volume to obtain geometry features

of stereo branch GjS  follow IGEV. Concatenated with FjS

and DjS, we obtain the stereo condition as:

xjS = [Eng ([GjS , FjS , DjS]), End (DjM), DjM],     (3)

where Eng and End are two convolutional layers for feature encoding. We feed xjS into condition-guided ConvGRUs to update the hidden state him-1  of monocular branch as:

zj  = σ(Conv([hjM-1, xjS], Wz ) + ck ),

(4)

rj  = σ(Conv([hjM-1, xjS], Wr ) + cr ),

˜jh M = tanh(Conv([rj  Θ hjM-1, xjS], Wh ) + ch ), hjM = (1 - zj ) Θ hjM-1 + zj  Θ ˜jh M ,

where ck , cr , ch  are context features. Based on the hidden state hjM, we decode a residual shift ∆t through two con- volutional layers to update the monocular disparity:

Dj+1M  = DjM  + ∆t.                     (5)

Mono Guided Refinement (MGR). Symmetric to SGA, MGR leverages the aligned monocular disparity to address stereo deficiencies in ill-posed regions, thin structures, and distant objects. Specifically, we employ the same condition- guided GRU architecture with independent parameters to refine stereo disparity. We simultaneously calculate the flow

residual maps FjM , FjS and geometric features GjM , GjS for

both the monocular and stereo branches, providing a com- prehensive stereo refinement guidance:

xjM = [Eng ([GjM , FjM , DjM]), End (DjM), DjM ,     (6)

Eng ([GjS , FjS , DjS]), End (DjS), DjS],

Left Image                                                      IGEV                                                       MonSter

Figure 5. Qualitative results on ETH3D. MonSter outperforms IGEV in challenging areas with strong reflectance, fine structures etc.

where  GjM   is  the  geometry  features  of  the  monocular

branch obtained by indexing from the Geometry Encoding

Volume using the monocular disparity DjM . xjM represents

the monocular condition for ConvGRUs,  and we use the

Eq. (4) to update the hidden state hiS  similarly, with only

the condition input changed from xiS  to xiM .  We use the

same two convolutional layers to decode the residual dis- parity △d and update the current stereo disparity following Eq. (5). After N2 rounds of dual-branched refinement, the disparity of the stereo branch is the final output of MonSter.

## 3.3. Loss Function

We  use  the  L1  loss  to  supervise  the  output  from  two branches. We denote the set of disparities from the first N1 iterations of the stereo branch as {di }10-1  and follow [20] to exponentially increase the weights as the number of iter- ations increases. The total loss is defined as the sum of the monocular branch loss LMono  and the stereo branch loss LStereo as follows:

where γ = 0.9, and dgt is the ground truth.

# 4. Experiment

## 4.1. Implementation Details

We  implement  MonSter  with  PyTorch  and  perform  ex- periments using NVIDIA RTX 3090 GPUs.   We use the AdamW [21] optimizer and clip gradients to the range [-1, 1] following baseline [38].  We use the one-cycle learning rate schedule with a learning rate of 2e-4 and train Mon- Ster with a batch size of 8 for 200k steps as the pretrained model. For the monocular branch, we use the ViT-large ver- sion of DepthAnythingV2 [46] and freeze its parameters to prevent training of stereo-matching tasks from affecting its generalization and accuracy.

Following the standard [16, 17, 35], we pretrain Mon- Ster on Scene Flow [22] for most experiments.  For fine- tuning on ETH3D and Middlebury, we follow SOTA meth- ods  [16, 17, 35] to create the Basic Training  Set  (BTS) from  various  public  datasets  for  pretraining,   including Scene Flow [22], CREStereo [16], Tartan Air [34], Sintel Stereo [3], FallingThings [32] and InStereo2k [2].

## 4.2. Benchmark Performance

To demonstrate the outstanding performance of our method, we  evaluate  MonSter  on  the  five  most  commonly  used benchmarks:  KITTI 2012 [11], KITTI 2015 [23], ETH3D [28], Middlebury [27] and Scene Flow [22].

Scene Flow [22]. As shown in Tab. 1, we achieve a new state-of-the-art performance with an EPE metric of 0.37 on Scene Flow, surpassing our baseline [38] by 21.28% and outperforming the SOTA method [35] by 15.91%.

ETH3D[28]. Following the SOTA methods [16, 17, 35,

| Method | GwcNet[14] | LEAStereo[8] | ACVNet[37] | IGEV[38] | Selective-IGEV[35] | MonSter (Ours) |
| --- | --- | --- | --- | --- | --- | --- |
| EPE (px)↓ | 0.76 | 0.78 | 0.48 | 0.47 | 0.44 | 0.37 (-15 .91%) |

Table 1. Quantitative evaluation on Scene Flow test set. The best result is bolded, and the second-best result is underscored.

| Method | ETH3D[28] |  |  | Middlebury[27] |  |  | KITTI 2015[11] |  |  |  | KITTI 2012[11] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bad1.0 Bad1.0 RMSE Noc All Noc |  |  | Bad2.0 Bad2.0 RMSE Noc All Noc |  |  | D1-bg Noc | D1-all Noc | D1-bg All | D1-all All | Out-2 Noc | Out-2 All | Out-3 Noc | Out-3 All |
| GwcNet[14] | 6.42 | 6.95 | 0.69 | - | - | - | 1.61 | 1.92 | 1.74 | 2.11 | 2.16 | 2.71 | 1.32 | 1.70 |
| GANet[48] | 6.22 | 6.86 | 0.75 | - | - | - | 1.40 | 1.73 | 1.55 | 1.93 | 1.89 | 2.50 | 1.19 | 1.60 |
| LEAStereo[8] | - | - | - | 7.15 | 12.10 | 8.11 | 1.29 | 1.51 | 1.40 | 1.65 | 1.90 | 2.39 | 1.13 | 1.45 |
| ACVNet[37] | 2.58 | 2.86 | 0.45 | 13.70 | 19.50 | 32.2 | 1.26 | 1.52 | 1.37 | 1.65 | 1.83 | 2.35 | 1.13 | 1.47 |
| RAFT-Stereo[20] | 2.44 | 2.60 | 0.36 | 4.74 | 9.37 | 8.41 | 1.44 | 1.69 | 1.58 | 1.82 | 1.92 | 2.42 | 1.30 | 1.66 |
| CREStereo[16] | 0.98 | 1.09 | 0.28 | 3.71 | 8.13 | 7.70 | 1.33 | 1.54 | 1.45 | 1.69 | 1.72 | 2.18 | 1.14 | 1.46 |
| IGEV[38] | 1.12 | 1.51 | 0.34 | 4.83 | 8.16 | 12.80 | 1.27 | 1.49 | 1.38 | 1.59 | 1.71 | 2.17 | 1.12 | 1.44 |
| CroCo-Stereo[36] | 0.99 | 1.14 | 0.30 | 7.29 | 11.11 | 8.91 | 1.30 | 1.51 | 1.38 | 1.59 | - | - | - | - |
| DLNR[49] | - | - | - | 3.20 | 6.98 | 7.78 | 1.42 | 1.61 | 1.60 | 1.76 | - | - | - | - |
| Selective-IGEV[35] | 1.23 | 1.56 | 0.29 | 2.51 | 6.04 | 7.26 | 1.22 | 1.44 | 1.33 | 1.55 | 1.59 | 2.05 | 1.07 | 1.38 |
| LoS[17] | 0.91 | 1.03 | 0.31 | 4.20 | 8.03 | 6.99 | 1.29 | 1.52 | 1.42 | 1.65 | 1.69 | 2.12 | 1.10 | 1.38 |
| NMRF-Stereo[13] | - | - | - | - | - | - | 1.18 | 1.46 | 1.28 | 1.57 | 1.59 | 2.07 | 1.01 | 1.35 |
| MonSter(Ours) | 0.46 | 0.72 | 0.20 | 2.64 | 6.14 | 6.71 | 1.05 | 1.33 | 1.13 | 1.41 | 1.36 | 1.75 | 0.84 | 1.09 |

Table 2.  Results on four popular benchmarks.  All results are derived from official leaderboard publications or corresponding papers. All metrics are presented in percentages, except for RMSE, which is reported in pixels. For testing masks, “All” denotes being tested with all pixels while “Noc” denotes being tested with a non-occlusion mask. The  best   and   second best  are marked with colors.

40, 42], the full training set is composed of the BTS and ETH3D training set. Our MonSter ranks 1st on the ETH3D leaderboard. As shown in Tab. 2, MonSter achieves the best performance among all published methods for all metrics. Compared with baseline IGEV, we achieve improvements of 58.93%, 52.32%, and 41.18% in the three reported met- rics, respectively.  Qualitative comparisons shown in Fig.5 exhibit a similar trend.  Notably, even compared with the previous best method LoS, we improved the Bad 1.0 (Noc) metric from 0.91 to 0.46, achieving a 49.45% improvement.

Middlebury. Also following  [16, 17, 35, 42], the train- ing set of Middlebury is the combination of the BTS and Middlebury training set.   As  shown in Tab.   2, MonSter outperforms all existing methods in terms of RMSE metric. Compared with baseline [38], we achieved an improvement of 45.34% on the Bad 2.0 (Noc) metric.

KITTI. Following the training of SOTA methods   [13, 16, 35], we finetuned Scene Flow pretrained model on the mixed dataset of KITTI 2012 and KITTI 2015 for 50k steps. At the time of writing, MonSter ranks 1st  simultaneously on both the KITTI 2012 and KITTI 2015 leaderboards. As shown in Tab.  2, we achieve the best performance for all metrics.   On KITTI 2015, MonSter surpasses CREStereo [16] and Selective-IGEV [35] by 16.57% and 9.03% on D1- all metric of all regions, respectively.  As for KITTI 2012, we significantly outperform the existing SOTA by a large margin. Compared to the current highest-precision method, NMRF-Stereo [13] and Selective-IGEV [35], we achieve a 19.26% improvement and a 21.01% improvement respec-

| Method | KITTI 2012 Reflective Region |
| --- | --- |
|  | Out-2 Out-2 Out-3 Out-3 Out-4 Out-4 Noc All Noc All Noc All |
| ACVNet[37] CREStereo[16] IGEV[38] Selective-IGEV[35] LoS[17] NMRF-Stereo[13] MonSter(Ours) | 11.42 13.53 7.03 8.67 5.18 6.48 9.71 11.26 6.27 7.27 4.93 5.55 7.57 8.80 4.35 5.00 3.16 3.57 6.73 7.84 3.79 4.38 2.66 3.05 6.31 7.84 3.47 4.45 2.41 3.01 10.02 12.34 6.35 8.11 4.80 6.09 5.66 6.81 2.75 3.38 1.73 2.13 |

Table 3. Results of the reflective regions on KITTI 2012 leader- board. The  best  and   second best  are marked with colors.

| Method | Edges |  | Non-Edges |  |
| --- | --- | --- | --- | --- |
|  | EPE | >1px | EPE | >1px |
| IGEV [38] | 2.23 | 20.42 | 0.41 | 4.58 |
| Selective-IGEV [35] | 2.18 | 20.01 | 0.38 | 4.35 |
| MonSter(Ours) | 1.91 | 18.59 | 0.31 | 3.57 |

Table 4. Comparison on SceneFlow test set in different regions. tively in the Out-3 metric across all regions.

## 4.3. Performance in Ill-posed Regions

MonSter fully leverages the advantages of monocular depth priors,  effectively overcoming challenges in ill-posed re- gions.   To validate this, we conduct comparisons on sev- eral representative ill-posed regions, such as reflective ar- eas, edge and non-edge regions, and distant backgrounds:

| Method | KITTI-12 KITTI-15 Middlebury ETH3D (>3px) (>3px) (>2px) (>1px) |  |  |  |
| --- | --- | --- | --- | --- |
| Training Set | Scene Flow |  |  |  |
| CFNet[29] | 4.72 | 5.81 | 9.80 | 5.80 |
| RAFT-Stereo[20] | 5.12 | 5.74 | 9.36 | 3.28 |
| CREStereo[16] | 5.03 | 5.79 | 12.88 | 8.98 |
| Selective-IGEV[35] | 5.64 | 6.05 | 12.04 | 5.40 |
| NMRF-Stereo[13] | 4.23 | 5.10 | 7.54 | 3.82 |
| IGEV[38] | 4.84 | 5.51 | 6.23 | 3.62 |
| MonSter(Ours) | 3.62 | 3.97 | 5.17 | 2.03 |

| Training Set Scene Flow + CREStereo + Tartan Air |  |  |  |  |
| --- | --- | --- | --- | --- |
| IGEV[38] MonSter(Ours) | 3.95 | 4.30 | 5.56 | 2.38 |
|  | 2.95 | 3.23 | 2.94 | 1.21 |

Table 5.  Zero-shot generalization benchmark.  The top half is generalization comparisons only trained on Scene Flow. The bot- tom half compares generalizations for models trained on a combi- nation of Scene Flow, CREStereo, and TartanAir datasets.

•  Reflective Regions:  We conducted comparisons on the reflective regions of KITTI 2012 benchmark.   MonSter ranks 1st  on the KITTI 2012 leaderboard for all metrics of reflective regions.  As shown in Tab.3, we have ele- vated the existing SOTA to a new level.   MonSter sur- passes Selective-IGEV and LoS by 30.16% and 29.24% on Out-4(All) metric, respectively. Notably, compared to the SOTA method NMRF-Stereo, we achieved significant improvements of 58.32% and 65.02% in the Out-3(All) and Out-4(All) metrics respectively.

•  Edge & Non-edge Regions: Stereo matching faces chal- lenges in edge and low-texture regions, which our method addresses by leveraging the strengths of monocular depth. To evaluate MonSter in these areas, we divide the Scene Flow test set into edge and non-edge regions using the Canny operator following [35]. As shown in Tab. 4, Mon- Ster outperforms the baseline [38] by 14.35% and 24.39% in edge regions and non-edge regions.  Even when com- pared to [35], which is specifically designed to address edge and textureless regions,  our method achieves im- provements of 12.39% and  18.42% in two types of re- gions on EPE metric, respectively.

•  Distant Backgrounds:  Stereo matching struggles with depth perception for distant objects, we improve it by in- corporating our SGA and MGR modules.  As shown in Tab.2, MonSter improves the D1-bg metric by  18.12% compared to the baseline on KITTI 2015 benchmark. The D1-bg metric reflects the percentage of stereo disparity outliers averaged specifically over background regions.

## 4.4. Zero-shot Generalization

Our method not only improves the accuracy of in-domain datasets but also enhances generalization ability. We evalu- ate the generalization performance of MonSter from syn- thetic  datasets  to  unseen  real-world  scenes.     All  mod-

Left Image                         IGEV                           MonSter

Figure 6. Visual comparisons on our captured real-world zero- shot data provide a more comprehensive assessment of gener- alization capability. All models are trained solely on the synthetic SceneFlow dataset.  MonSter significantly outperforms IGEV in textureless regions, reflective areas and fine structures, etc.

els were trained on the same synthetic datasets and then tested  directly  on  real-world  datasets:    KITTI[11,  23], Middlebury[27],  and ETH3D[28] training sets.   We con- ducted a more thorough generalization comparison by train- ing with different datasets:

Scene Flow: We compared with SOTA methods which are only trained on the Scene Flow training set. As shown in the top half of Tab. 5, MonSter achieves the best gener- alization performance across four datasets.  MonSter even surpasses CFNet and CREStereo, both of which are specif- ically designed for cross-domain generalization.

Mix 3 datasets:  Interestingly, only adding CREStereo and TartanAir to the training set, the generalization of Mon- Ster improves significantly. This is because the SceneFlow dataset differs substantially from real-world distributions, whereas the virtual data in TartanAir more closely resem- ble real-world scenes.  This similarity helps our SGA and MGR modules better in learning how to effectively inte- grate monocular and stereo cues.  As shown in the bottom half of Tab. 5, we surpass the baseline[38] by 47.12% and 49.16% on Middlebury and ETH3D respectively.

The  strong  sim-to-real  generalization  of  MonSter  is highly encouraging.  In the future, we plan to scale up the size and diversity of simulation training data to further boost the performance of MonSter.

## 4.5. Ablation Study

To  demonstrate  the  effectiveness  of  each  component  of MonSter, we ablation on Scene Flow [22] and the following strategies are discussed:

•  Disparity Fusion:  Monocular depth often carries sub-

| Model | Monocular Depth | Disparity Fusion | Scale & Shift Refinement | Feature Sharing | EPE (px) | >1px (%) | Run-time (S) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline (IGEV) |  |  |  |  | 0.47 | 5.21 | 0.37 |
| Mono+Conv Mono+MGR Mono+MGR+Conv Mono+MGR+SGA Full model (MonSter) | ✓ ✓ ✓ ✓ ✓ | Conv MGR MGR MGR MGR | Conv SGA SGA | ✓ | 0.46 0.43 0.42 0.39 0.37 | 5.12 4.96 4.82 4.43 4.25 | 0.64 0.65 0.65 0.66 0.64 |

Table 6. Ablation study of the effectiveness of proposed modules on the Scene Flow test set.

| Model | Mono Depth Model | Iteration Number | Sceneflow (EPE) | Run-time (S) |
| --- | --- | --- | --- | --- |
| IGEV[38] |  | 32 | 0.47 | 0.37 |
| Full model | DepthAnythingV2[46] | 32 | 0.37 | 0.64 |
| Full model-4iter | DepthAnythingV2[46] | 4 | 0.42 | 0.34 |
| Full model-V1 | DepthAnythingV1[45] | 32 | 0.39 | 0.64 |
| Full model-midas | MiDaS[25] | 32 | 0.41 | 0.51 |

Table 7. Efficiency and universality of MonSter.

stantial noise, which complicates its fusion with the stereo branch’s disparity.  To demonstrate the efficiency of our Mono Guided Refinement (MGR) fusion method, we re- placed the MGR module with a convolution-based hour- glass network of equal parameter numbers, directly con- catenating the monocular and  stereo disparities for fu- sion, denoted as Mono+Conv.  As shown in Tab.6, com- pared with  ‘Mono+Conv’,  ‘Mono+MGR’  improves the EPE metric by 6.52%.

•  Scale  &  Shift  Refinement:   As shown in Fig.3,  even after global scale-shift alignment, the monocular depth still suffers from significant scale and shift ambiguities. Therefore, leveraging high-confidence regions from the stereo branch to correct the scale and shift of the monoc- ular disparity is essential.   We incorporated our Stereo Guided Refinement module (SGA) to optimize the scale and shift based on  ‘Mono+MGR’. As  shown in Tab.6, compared with ‘Mono+MGR’, our SGA module achieves improvements of 9.30% in EPE and  10.69% in  1-pixel error, demonstrating that monocular depth with accurate scale and shift can provide further enhancements to stereo matching.   Similarly,  we validated that our SGA mod- ule is more effective than direct convolution-based fu- sion.  Compared to  ‘Mono+MGR+Conv’, our SGA still achieves an 8.09% improvement in 1-pixel error.

•  Feature Sharing:  The feature extraction component of monocular depth estimation models, pre-trained on large datasets, contains rich contextual information that can be shared with the stereo branch. We employ a feature trans- fer network to map these features, transforming them into a representation compatible with  stereo matching.   As shown in the last 2 rows of Tab.6, feature sharing im- proved the EPE metric by 5.13%.

Compatibility  with  Other  Monocular  Models.    To

validate the generalizability of MonSter to different depth models,  we replace the monocular branch with the ViT- large version of [45] and the dpt beit large version of Mi- DaS [25].  As shown in Tab.7, all MonSter variants con- sistently outperform the baseline [38].  This demonstrates the versatility of MonSter and its potential to benefit from future advancements in monocular depth models.

Efficiency. We use the ViT-large version of DepthAny- thingV2  [46] as the monocular branch, which introduces time  and  memory  overheads.     Compared  to  the  base- line [38], our inference time increases from 0.37s to 0.64s. With the monocular branch containing 335.3M parameters, the stereo branch having 12.6M parameters, and the SGA and MGR modules having 8.2M parameters, our full model has a total of 356.1M parameters. Existing methods [1, 36] also use ViT-based encoders with a large number of param- eters. For example, CroCo-Stereo [36] has 437.4M parame- ters, larger than MonSter, yet its performance is inferior, as shown in Tab.2, this demonstrates that the number of pa- rameters does not determine the final effectiveness.   Our research focuses on the accuracy and generalization capa- bilities, given the significant improvement in accuracy and generalization (as shown in Tab.2and Tab.5), the additional inference cost is acceptable. Notably, our approach simpli- fies stereo matching and enables us to achieve SOTA per- formance with only 4 iterations (N1 =N2 =2).  As shown in Tab.7, while the baseline requires 32 iterations, our method achieves a 10.64% improvement in EPE with just 4 itera- tions, achieving a better accuracy-speed trade-off.  Further memory reduction can be achieved through encoder quanti- zation or distillation, which we leave as future work.

# 5. Conclusion

We propose MonSter, a novel pipeline that decouples the stereo  matching  task  into  a  simpler  paradigm  of recov- ering scale and shift from relative depth.   This  approach fully  leverages  the  contextual  and  geometric priors pro- vided by monocular methods while avoiding issues such as noise and scale ambiguity.  As a result, our method sig- nificantly enhances the accuracy and robustness of stereo matching in ill-posed regions. MonSter achieves substantial improvements over the state-of-the-art on five widely used

benchmarks.  Additionally, MonSter also demonstrates the best generalization performance, and experimental results show that there is still considerable room for improvement. Therefore, our future work will focus on scaling up Mon- Ster to serve as a stereo foundation model, enabling a wide range of downstream applications that require metric depth.

References

[1]  Jihye  Ahn,   Hyesong  Choi,   Soomin  Kim,   and  Dongbo Min.  Madis-stereo:  Enhanced stereo matching via distilled masked image modeling.  arXiv preprint arXiv:2409.02846, 2024. 8

[2]  Wei Bao, Wei Wang, Yuhua Xu, Yulan Guo, Siyu Hong, and Xiaohu Zhang.   Instereo2k:  a large real dataset for  stereo matching in indoor scenes.  Science China Information Sci- ences, 63:1–11, 2020. 5

[3]  Daniel   J  Butler,   Jonas  Wulff,   Garrett   B  Stanley,   and Michael J Black.  A naturalistic open source movie for opti- cal flow evaluation.  In Computer Vision–ECCV 2012: 12th European Conference on Computer Vision, Florence, Italy, October 7-13, 2012, Proceedings, Part VI 12, pages 611– 625. Springer, 2012. 5

[4]  Jia-Ren  Chang  and  Yong-Sheng  Chen.     Pyramid  stereo matching network.   In CVPR, pages 5410–5418, 2018.  1, 2, 3

[5]  Junda Cheng, Xin Yang, Yuechuan Pu, and Peng Guo.  Re- gion separable stereo matching.  IEEE Transactions on Mul- timedia, 25:4880–4893, 2022. 3

[6]  Junda Cheng, Gangwei Xu, Peng Guo, and Xin Yang.  Coa- trsnet: Fully exploiting convolution and attention for stereo matching by region  separation.    International  Journal  of Computer Vision, 132(1):56–73, 2024. 3

[7]  Junda Cheng, Wei Yin, Kaixuan Wang, Xiaozhi Chen, Shijie Wang, and Xin Yang.   Adaptive fusion of single-view and multi-view depth for autonomous driving. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pages 10138–10147, 2024. 3

[8]  Xuelian  Cheng,  Yiran  Zhong,  Mehrtash Harandi,  Yuchao Dai, Xiaojun Chang, Hongdong Li, Tom Drummond, and Zongyuan Ge.   Hierarchical neural  architecture search for deep stereo matching. NeurIPS, 33:22158–22169, 2020. 6

[9]  Kyunghyun  Cho, Bart Van Merrinboer,  Caglar Gulcehre, Dzmitry Bahdanau, Fethi Bougares, Holger Schwenk, and Yoshua Bengio.  Learning phrase representations using rnn encoder-decoder for statistical machine translation.   arXiv preprint arXiv:1406.1078, 2014. 2

[10]  Miaojie Feng, Junda Cheng, Hao Jia, Longliang Liu, Gang- wei Xu, and Xin Yang.  Mc-stereo:  Multi-peak lookup and cascade search range for stereo matching.   In 2024 Inter- national Conference on 3D Vision (3DV), pages 344–353. IEEE, 2024. 2

[11]  Andreas Geiger, Philip Lenz, and Raquel Urtasun.  Are we ready for autonomous driving?  the kitti vision benchmark suite. In CVPR, pages 3354–3361. IEEE, 2012. 2, 5, 6, 7

[12]  Xiaodong Gu, Zhiwen Fan, Siyu Zhu, Zuozhuo Dai, Feitong Tan, and Ping Tan.  Cascade cost volume for high-resolution

multi-view stereo and stereo matching.   In Proceedings of the IEEE/CVF conference on computer vision and pattern recognition, pages 2495–2504, 2020. 3

[13]  Tongfan  Guan,   Chen  Wang,  and  Yun-Hui  Liu.     Neural markov random field for stereo matching. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pages 5459–5469, 2024. 6, 7

[14]  Xiaoyang Guo, Kai Yang, Wukui Yang, Xiaogang Wang, and Hongsheng Li.  Group-wise correlation stereo network.  In CVPR, pages 3273–3282, 2019. 1, 3, 6

[15]  Alex Kendall, Hayk Martirosyan, Saumitro Dasgupta, Peter Henry, Ryan Kennedy, Abraham Bachrach, and Adam Bry. End-to-end learning of geometry and context for deep stereo regression. In ICCV, pages 66–75, 2017. 1

[16]  Jiankun  Li, Peisen Wang, Pengfei Xiong, Tao Cai, Ziwei Yan, Lei Yang, Jiangyu Liu, Haoqiang Fan, and Shuaicheng Liu.   Practical stereo matching via cascaded recurrent net- work with adaptive correlation.    In  CVPR, pages  16263– 16272, 2022. 1, 2, 3, 5, 6, 7

[17]  Kunhong  Li,  Longguang  Wang,  Ye  Zhang,  Kaiwen  Xue, Shunbo Zhou, and Yulan Guo.  Los: Local structure-guided stereo matching.  In Proceedings of the IEEE/CVF Confer- ence on  Computer  Vision and Pattern Recognition,  pages 19746–19756, 2024. 3, 5, 6

[18]  Zhaoshuo Li, Xingtong Liu, Nathan Drenkow, Andy Ding, Francis X Creighton,  Russell H Taylor,  and Mathias Un- berath. Revisiting stereo depth estimation from a sequence- to-sequence perspective with transformers.  In Proceedings of the IEEE/CVF international conference on computer vi- sion, pages 6197–6206, 2021. 2

[19]  Haotong Lin, Sida Peng, Jingxiao Chen, Songyou Peng, Ji- aming Sun, Minghuan Liu, Hujun Bao, Jiashi Feng, Xiaowei Zhou, and Bingyi Kang.  Prompting depth anything for 4k resolution accurate metric depth estimation.  arXiv preprint arXiv:2412.14015, 2024. 3

[20]  Lahav  Lipson, Zachary Teed,  and Jia Deng.   Raft-stereo: Multilevel recurrent field transforms for stereo matching.  In 3DV, pages 218–227. IEEE, 2021. 1, 2, 3, 5, 6, 7

[21]  Ilya Loshchilov and Frank Hutter.  Decoupled weight decay regularization. arXiv preprint arXiv:1711.05101, 2017. 5

[22]  Nikolaus Mayer, Eddy Ilg, Philip Hausser, Philipp Fischer, Daniel Cremers, Alexey Dosovitskiy, and Thomas Brox.  A large dataset to train convolutional networks for disparity, optical flow,  and scene flow estimation.   In  CVPR,  pages 4040–4048, 2016. 1, 2, 3, 5, 7

[23]  Moritz Menze and Andreas Geiger.   Object scene flow for autonomous vehicles. In CVPR, pages 3061–3070, 2015. 2, 5, 7

[24]  Maxime Oquab, Timothe Darcet, Tho Moutakanni, Huy Vo,  Marc  Szafraniec,   Vasil  Khalidov,   Pierre  Fernandez, Daniel Haziza, Francisco Massa, Alaaeldin El-Nouby, et al. Dinov2: Learning robust visual features without supervision. arXiv preprint arXiv:2304.07193, 2023. 3

[25]  Ren   Ranftl,   Katrin   Lasinger,   David   Hafner,   Konrad Schindler, and Vladlen Koltun.  Towards robust monocular depth estimation: Mixing datasets for zero-shot cross-dataset transfer.  IEEE transactions on pattern analysis and machine intelligence, 44(3):1623–1637, 2020. 8

[26]  Ren Ranftl, Alexey Bochkovskiy, and Vladlen Koltun.  Vi- sion transformers for dense prediction.   In Proceedings of the IEEE/CVF international conference on computer vision, pages 12179–12188, 2021. 3

[27]  Daniel   Scharstein,   Heiko  Hirschm¨uller,   York   Kitajima, Greg Krathwohl, Nera Nei, Xi Wang,  and Porter West- ling.  High-resolution stereo datasets with subpixel-accurate ground truth.  In GCPR, pages 31–42. Springer, 2014. 2, 5, 6, 7

[28]  Thomas Schops, Johannes L Schonberger, Silvano Galliani, Torsten Sattler, Konrad Schindler, Marc Pollefeys, and An- dreas Geiger.   A  multi-view  stereo benchmark with high- resolution images and multi-camera videos. In CVPR, pages 3260–3269, 2017. 2, 5, 6, 7

[29]  Zhelun Shen, Yuchao Dai, and Zhibo Rao.  Cfnet:  Cascade and fused cost volume for robust stereo matching. In CVPR, pages 13906–13915, 2021. 3, 7

[30]  Zhelun Shen, Yuchao Dai, Xibin Song, Zhibo Rao, Dingfu Zhou, and Liangjun Zhang.  Pcw-net: Pyramid combination and warping cost volume for stereo matching.  In European conference on computer vision,  pages 280–297.  Springer, 2022. 3

[31]  Xiao Song, Xu Zhao, Liangji Fang, Hanwen Hu, and Yizhou Yu. Edgestereo: An effective multi-task learning network for stereo matching and edge detection.  IJCV, 128(4):910–930, 2020. 2, 3

[32]  Jonathan Tremblay, Thang To, and Stan Birchfield.  Falling things: A synthetic dataset for 3d object detection and pose estimation.     In  Proceedings  of  the  IEEE  Conference  on Computer Vision and Pattern Recognition Workshops, pages 2038–2041, 2018. 5

[33]  A Vaswani.  Attention is all you need.  Advances in Neural Information Processing Systems, 2017. 2

[34]  Wenshan Wang, Delong Zhu, Xiangwei Wang,  Yaoyu Hu, Yuheng Qiu, Chen Wang, Yafei Hu, Ashish Kapoor, and Se- bastian Scherer.  Tartanair:  A dataset to push the limits of visual slam.   In  2020 IEEE/RSJ International  Conference on Intelligent Robots and Systems (IROS), pages 4909–4916. IEEE, 2020. 5

[35]  Xianqi  Wang,   Gangwei   Xu,   Hao  Jia,   and   Xin  Yang. Selective-stereo:  Adaptive frequency information selection for stereo matching.  In Proceedings of the IEEE/CVF Con- ference on Computer Vision and Pattern Recognition, pages 19701–19710, 2024. 1, 2, 3, 5, 6, 7

[36]  Philippe   Weinzaepfel,    Thomas   Lucas,   Vincent   Leroy, Yohann Cabon, Vaibhav Arora, Romain Brgier,  Gabriela Csurka,  Leonid  Antsfeld,  Boris  Chidlovskii,  and  Jrme Revaud.   Croco v2:  Improved cross-view completion pre- training for stereo matching and optical flow. In Proceedings of the IEEE/CVF International Conference on Computer Vi- sion, pages 17969–17980, 2023. 6, 8

[37]  Gangwei Xu, Junda Cheng, Peng Guo, and Xin Yang. Atten- tion concatenation volume for accurate and efficient stereo matching. In CVPR, pages 12981–12990, 2022. 1, 2, 3, 6

[38]  Gangwei Xu, Xianqi Wang, Xiaohuan Ding, and Xin Yang. Iterative geometry encoding volume for stereo matching.  In Proceedings of the IEEE/CVF Conference on Computer Vi-

sion and Pattern Recognition, pages 21919–21928, 2023. 1, 2, 3, 5, 6, 7, 8

[39]  Gangwei Xu, Yun Wang, Junda Cheng, Jinhui Tang, and Xin Yang.  Accurate and efficient stereo matching via attention concatenation volume.  IEEE Transactions on Pattern Anal- ysis and Machine Intelligence, 2023. 1

[40]  Gangwei Xu, Xianqi Wang, Zhaoxing Zhang, Junda Cheng, Chunyuan Liao, and Xin Yang. Igev++: iterative multi-range geometry  encoding  volumes  for  stereo  matching.    arXiv preprint arXiv:2409.00638, 2024. 6

[41]  Haofei Xu and Juyong Zhang.  Aanet: Adaptive aggregation network for efficient stereo matching. In CVPR, pages 1959– 1968, 2020. 1

[42]  Haofei  Xu,  Jing  Zhang,  Jianfei  Cai,  Hamid  Rezatofighi, Fisher Yu, Dacheng Tao, and Andreas Geiger. Unifying flow, stereo and depth estimation.  IEEE Transactions on Pattern Analysis and Machine Intelligence, 2023. 2, 6

[43]  Haofei  Xu,  Jing  Zhang,  Jianfei  Cai,  Hamid  Rezatofighi, Fisher Yu, Dacheng Tao, and Andreas Geiger. Unifying flow, stereo and depth estimation.  IEEE Transactions on Pattern Analysis and Machine Intelligence, 2023. 1, 3

[44]  Guorun  Yang,  Hengshuang  Zhao,  Jianping  Shi,  Zhidong Deng, and Jiaya Jia.  Segstereo: Exploiting semantic infor- mation for disparity estimation.  In ECCV, pages 636–651, 2018. 2, 3

[45]  Lihe Yang, Bingyi Kang, Zilong Huang, Xiaogang Xu, Jiashi Feng, and Hengshuang Zhao.  Depth anything:  Unleashing the power of large-scale unlabeled data.  In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pages 10371–10381, 2024. 8

[46]  Lihe Yang, Bingyi Kang, Zilong Huang, Zhen Zhao, Xiao- gang Xu, Jiashi Feng, and Hengshuang Zhao.   Depth any- thing v2.  arXiv preprint arXiv:2406.09414, 2024.  2, 3, 5, 8

[47]  Chenghao Zhang, Gaofeng Meng, Bing Su, Shiming Xiang, and Chunhong Pan.    Monocular contextual constraint for stereo matching with adaptive weights assignment.   Image and Vision Computing, 121:104424, 2022. 3

[48]  Feihu   Zhang,    Victor   Prisacariu,    Ruigang   Yang,    and Philip HS Torr.   Ga-net:  Guided  aggregation net for end- to-end stereo matching.  In CVPR, pages 185–194, 2019. 1, 6

[49]  Haoliang Zhao,  Huizhou Zhou, Yongjun Zhang, Jie Chen, Yitong Yang, and Yong Zhao. High-frequency stereo match- ing network. In Proceedings of the IEEE/CVF conference on computer vision and pattern recognition, pages 1327–1336, 2023. 2, 3, 6